P2C90 Fix:

Install:

- Install 2.5” coupler off passenger side turbo.

- Locate the ½” hose coming off the driverside valve cover and going to the intake driver side off the turbo. Cut the hose and add the provided T.

- Install the 1” to ½” barbed fitting into the coupler that you installed on the passenger side

- Using the provided hose, connect the ½” barbed fitting to the T fitting on the driver side.

- With the provided zipties, tighten down each connection.